High Performance Liquid Chromatography (HPLC)

Make			: Waters, USA

Model			: Waters 2996 with 515 reciprocating pump and system controller

Specification 		: Binary HPLC System with PDA detector

Working Principle 	: Chromatography is basically a separation technique being used for 			separating complex mixture of compounds into their individual 				components based on the physical and chemical properties. 				Separation of compounds essentially requires a Mobile (solvent) and 			Stationary phase (immobile material). The mixture of analytes 				travels across the Stationary phase with mobile phase and based 				upon the polarity, charges, molecular weight and functional groups, 			get separated from the mixture. HPLC is a high resolution 					chromatographic method which involves the Stationary phase of very 			small particle size (~5µm) which does not allow mobile phase 				containing analytes to pass through without high pressure. Under 				given conditions, the analysis remains efficient, reliable and 				reproducible for most of the compounds therefore, the instrument is 			inevitable for the qualitative and quantitative targeted metabolite 			characterization and profiling in plants, microbes and other 				biological samples. Photodiode array detector attached with the 				system projects 3-D absorption over UV-VIS range and therefore, 				improves accuracy and efficiency of detection.

Major Features  	: Dual pump HPLC coupled with PDA detector coupled with C-8, C-18, 			Amino columns and EMPOWER software for data generation, suitable 			for isocratic and gradient analysis of plant and microbial samples for 			secondary metabolic profiling, qualitative characterization of 				compounds and quantitative analysis of targeted metabolites.

Applications 		: Qualitative metabolite profiling, Quantitative analysis of compounds 			of interest, Impurity check, Metabolites of herbal and food products, 			Pharmacokinetics applications, Plant and microbial 						metabolite studies

Prior requirements 	: Nature of the samples, information about the running conditions, 			source of compound and mixture, chemical standards, UV spectra of 			the metabolite or mixture (200-700 nm)

Charges (INR) 		:

| S. No. | Operating Conditions | Charges (Rs.) |  |
| --- | --- | --- | --- |
|  |  | Academic users | Industry users |
| 1. | Isocratic run for single compound or mixture (< 20 min) | 400 | 600 |
| 2. | Isocratic run for metabolite/mixture (<20 min) | 600 | 800 |
| 3. | Gradient run for metabolite mixture (<30 min) | 800 | 1000 |
| 4. | Gradient run for metabolite mix (>30 min) | 1200 | 1500 |

Terms & conditions 	: It is advisable to discuss about the samples and separation 				conditions prior to proceed for submission of samples for analysis. 			Loss or degradation of samples during transportation shall not be the 			responsibility of ICAR-NBAIM.

****